Exam of Digital Communications

a.a. 2019-2020

Nov. 25, 2019

Exercise #1

The random process  V(t) is defined by

V(t)=Xcos2πf0t+Ysin2πf0t

where X and Y are two zero-mean independent Gaussian random variables each with the variance

σ2.

- Find the expected value of V(t).

- Find the autocorrelation  RX(t+τ,t). Is V(t) stationary?

- Find the power spectral density of V(t)

- Find the power spectral density of the complex envelope of V(t)

- Find the in-phase and quadrature components of V(t)

Exercise #2

A stationary random process has an autocorrelation function given by RX(τ)=A22eτcos2πf0τ

We know that the random process never exceeds 6 in magnitude, e.g. assume that A=6.

- How many quantization levels are required to guarantee an SQNR of at least 60dB?

- Assuming that the signal is quantized to satisfy the condition above, and assuming the approximate bandwidth of the signal is W, what is the minimum required bandwidth for the transmission of a binary PCM signal based on this quantization scheme?

Exercise #3

A channel has a bandwidth B=1400 Hz.

1. Select a symbol rate and the order M of an M-PAM signal constellation to achieve a 9600 bps

signal transmission.

2. If a square-root raised cosine pulse is used for the transmitter pulse, select the roll-off factor. Assume that the channel has an ideal frequency-response characteristic.