Exam of Digital Communications

a.a. 2018-2019

Sept, 19 2019

Exercise #1

An SSB AM signal is generated by modulating an 800 kHz carrier by the message signal

m(t) = cos(2000πt) + 2 sin(2000πt). Assume that the amplitude of the carrier is Ac = 100.

(a) Determine the Hilbert transform of the modulated signal

(b) Determine the spectrum of the LSSB AM signal.

Exercise #2

The two pulses shown in the figure below are employed in a binary orthogonal signaling

scheme to be used in a digital communication system with AWGN channel with noise energy N0/2.

- Sketch carefully the optimum receiver (and the impulse response of the filters are included in the receiver).

- Assuming that a correlator receiver is employed, due to a timing error, the output is incorrectly sampled at t = 0.75T. What is the increase in the probability of a bit error, Pb,

compared to that of an ideal system?

Exercise #3

A voice signal in the range 3000 to 3300Hz is sampled at 8000 samples/s. We may transmit these samples directly as PAM pulses or we may first convert each sample to a PCM format and use binary (PCM) waveforms for transmission.

- What is the minimum system bandwidth required for the detection of PAM with no ISI and with a filter roll-off characteristics of r=1;

- Using the same roll-off, what is the minimum bandwidth required for the detection of binary (PCM) waveforms if the samples are quantized to eight levels?

- Repeat (b) using 128 quantization levels.

Exercise #4

A bandpass QPSK modulation system uses the following orthonormal signals:

ψ1(t)=110cos104πt     ψ2(t)=110sin104πt

The noise is AWGN with N0 = 1 and the target probability of a bit error is Pb = 10−5.

- Determine the maximum bit rate of the system (bps)

- The channel bandwidth is 450Hz and a raised cosine spectrum is employed to eliminate ISI. Determine the excess bandwidth and sketch carefully the spectrum

- Draw the constellation points

Exercise #5

Let us consider two channels that are modeled as linear filter and AWGN. The impulse responses of the two channels are drawn below:

- (b)

- In which case is more important to use an equalizer at the receiver?

- When the equalizer is not used, how do you expect that the BER curve look like?

- In case of low signal to noise ratio, would you use a zero-forcing or an MMSE equalizer?