Exam of Digital Communications

a.a. 2016-2017

July 28, 2017

Exercise #1

The spectrum of a lowpass signal is shown below.

- Determine the value of the Nyquist sampling rate Fs_min

- Sketch the ideal sampled spectrum when the sampling frequency is Fs=3Fs_min

- Write the output of an  ideal (rectangular) bandpass filter with f0=4, B=0.7 and gain ¼, when the input is the above mentioned sampled signal.

- Write the complex envelope of this output signal.

Exercise #2

Two equiprobable messages are transmitted via an AWGN channel with a noise power spectral density of N0/2=1. The messages are transmitted using the following two pulses (a=1, T=2):

- it is an antipodal or orthogonal modulation?

- Sketch an correlation type structure for the optimum receiver (defining the decision rule) and calculate the average probability of error.

- Let us assume that due to some imperfections, the correlator to the pulse s1 does not perform the integration between 0 and 1, but between 0 and 1.5. The second integrator works ideally. Compute the probability of error.

- What decision rule should be adopted to make an optimum receiver that takes into account that imperfection?

Exercise #3

A voice signal is sampled at a rate of 8kHz, logarithmically compressed and encoded into a PCM format using 8bits per sample. The PCM data is transmitted through an AWGN channl via M-level PAM. Determine the bandwidth required for the transmission when

- M=4;

- M=8;

- M=16.

Exercise #4

Suppose the QPSK is used for transmitting over an AWGN with a power spectral density of 1/2N0=10^(-10) W/Hz. The transmitted signal energy is Es=(1/2)A^(2)T, where T is the symbol interval and A is the signal amplitude. Determine the signal amplitude to achieve an error probability of 10^(-6) when the data rate is: i) 10kbps; ii) 100kbps; iii) 1Mbps.

Exercise #5

The following are the channel frequency responses of 3 different channels affected by ISI.

In the following figure there are the relative BER curves after MMSE equalization.

- Associate each BER curve with the proper channel frequency responses

- When the equalizer is not used, how do you expect that the BER curves look like?