Exam of Digital Communications

a.a. 2017-2018

Sept. 5, 2018

Exercise #1

A bandpass signal is given by

x(t) = sinc(2t) cos(3πt).

(a) Is the signal narrowband or wideband? Justify your answer.

(b) Find the complex baseband equivalent and sketch carefully its spectrum.

(a) Give an expression for the Hilbert transform of x(t)

Exercise #2

A binary communication system uses the following pulse:

s(t)=2ψ(t)    for bit '0'

where ψ(t) is a unit-energy rectangular pulse of duration is 3 seconds.

- it is an antipodal or orthogonal modulation?

- Determine the energy per bit, Eb.

- Sketch carefully a block diagram of the demodulator. In particular, you must give a detailed specification of the impulse response of the matched filter, the sampling time, and the decision rule

- Noise at the receiver is AWGN with  σ2=3. Determine the probability of a bit error, Pe

- By mistake, the transmitter sends the pulse shown in figure below. Determine the resulting probability of a bit error, Pe,mistake.

Exercise #3

A signal has a bandwidth of 4kHz. It is sampled, logarithmically compressed and encoded into a PCM format using 8bits per sample. The PCM data is transmitted through an AWGN channel via M-level PAM. Determine the minimum bandwidth required for the transmission when

- M=4;

- M=8;

- M=16.

Exercise #4

A bandpass communication system uses the constellation shown in the following Figure. The noise is AWGN with power spectral density N0/2.

- Calculate the energy per bit as a function of a

- Calculate the BER (assume Gray coding and not too low SNR)

- Compare the BER with the BER of a QPSK with the same energy per bit

Exercise #5

The following are the channel frequency responses of 3 different channels affected by ISI.

In the following figure there are the relative BER curves after MMSE equalization.

- Associate each BER curve with the proper channel frequency responses

- When the equalizer is not used, how do you expect that the BER curves look like?