p-

Exam of Digital Communications

a.a. 2020-2021

Feb. 24, 2021

Exercise #1

A message signal m (t) = cos 2000t + 2 cos 3000t modulates the carrier c (t) = 100 cos2fct, where fc = 1 MHz.

- Sketch the spectrum of the lower SSB modulated signal

- Find the complex envelope for the modulated signal

ct 50 E^2000pi ft + 100e^3000 pift

Exercise #2

A signal has a bandwidth of 8kHz. It is sampled, logarithmically compressed and encoded into a PCM format using 8bits per sample. The PCM data is transmitted through an AWGN channel via M-level PAM. Determine the minimum bandwidth required for the transmission when

- M=4;

- M=8;

- M=16.

Exercise #3

In an additive white Gaussian noise channel with noise power spectral density of N0/2, two bits are transmitted by

s1(t)=0s2(t)=A     0≤t≤T20    T2≤t≤T

The two messages are transmitted with a priori probabilities p and (1-p), respectively.

- Determine the average energy transmitted per bit Ealignlavb

- Determine the optimum threshold

- Determine the structure of the optimal receiver (including the detector after the sampler)

- Determine the probability of error in case p=(1-p)=0.5.

Exercise #4

If the maximum BER is 10(-4) and SNR is 15dB, which constellations guarantees the maximum bit rate? What is the maximum theoretical bit rate in case with no ISI if the bandwidth available is 3kHz?

Exercise #5

Let us consider two channels that are modeled as linear filter and AWGN. The impulse responses of the two channels are drawn below:

- (b)

- In which case is more important to use an equalizer at the receiver?

- When the equalizer is not used, how do you expect that the BER curve look like?